Terminal download instructions

For: Verifone Tranz 330/380/460

Make sure terminal is plugged into power and turned on.

Make sure an active, unshared telephone line is plugged into the terminal.

Make sure on the Tranz 380x2 that you have toggled to the side for the gift card by pressing the Enter key two times.

Press the clear key.

Press the function key then press the #8 key.

The terminal will ask for a password. Key in: 1AlphaAlpha66831, then press enter.

The terminal will request “store what?” Press the 0 (zero) key three times until an = appears at the end of the zeros, four times for a 380 or a 460.

Key in 18884188354N.  Press the 6 key and then the ‘alpha’ key two times to get the letter N. Press enter.

The terminal will request “store what?” Press 001 until the = appears. Press 0001 for 380/460.

Key in the download number (call World Gift Card to get). Press enter.

The terminal will request “store what?” a third time. Press 019 until the = appears. 0019 for 380/460.

Key in the letters FOOGP using the number keys and the Alpha key. Press enter.

Press clear.

To initialize the World Gift CardTM  Program download, press function, 0, function. The terminal will attempt to connect.

(if the terminal should request a download speed press the 2 and enter keys)

(on Tranz 380x2 the terminal will request a download type, 1 = 380, 2 = 330 – Note: the World Gift Card defaults to 330)

The program will download.

Follow the steps to use the Program.