Verifone Vx570 Quick Reference Guide (World Gift CardTM)

The OMNI Vx570 will work in any of these configurations – dialup (plain old telephone service) or Ethernet (broadband Internet, TCP/IP). Please be sure that the terminal is correctly installed for one of these choices.

Have all other applications to be used in the terminal (credit card, debit, check service) installed prior to calling for the gift card download. If you can, make note of the File Group numbers that were used (your ISO/credit card rep should be able to get you this information). Also make sure World Gift Card knows if your terminal will be processing through the internet or if it requires a code to get and outside line.

The red key is the ‘Cancel’ key, the yellow key is the ‘Back Space’ key and the green key is the ‘Enter’ key

Before beginning have them check it World Gift Card is already loaded in their terminal by having them press the Cancel key a few times and then the star key. Look for ‘World Gift Card’ in the display. If it is not there then proceed with the download.

### DOWNLOAD the World Gift Card Program

With the terminal plugged into power and the display at the idle screen, perform the following:

Press the F2 and F4 keys at the same time

Enter the password – 1 alpha alpha 6 6 8 3 1 (note ‘alpha’ is a key)

Press the key under the small arrow in the display pointing down until you see ‘Download’

Press the F2 key next to ‘Download’

Change the File Group number to 5 (or 6, depending on what programs are already installed in the terminal) and press Enter

Re-enter the password 1 alpha alpha 6 6 8 3 1, press Enter

Select Full (press the F3 key)

Select Modem (press the F2 key)

If the terminal has been downloaded before it may ask you to clear all files from Group 5.

Select ‘Continue’.

It will then display the current download parameters and if they need to be changed press ‘Edit’ otherwise press ‘Start’.

If you select edit you can enter new download parameters the same way as stated below. You can also change the download speed if needed, but usually you can just use the highest connection rate possible.

If the terminal has not been downloaded before then it should display ‘*ZP Host Phone Number’.

Enter 18662344598 and press ‘Enter’.

Next it will display ‘*ZT Terminal ID’.

Enter the download number for the particular merchant and press enter.

Lastly it will display ‘*ZA Application ID’.

Enter ‘WGC’ and press enter.

It will then give you a screen displaying the download info, if everything is correct press ‘Start’ to begin the download.

The terminal will indicate ‘Downloading Now’. Upon connection, a line will appear in the display, and asterisks will denote the progress of the download. The download will take a few minutes, then the terminal will display download done, the program will setup in the terminal and the terminal will return to the idle mode in the display World Gift Card should appear in the display. Press the * key to toggle the display to World Gift Card if necessary.